TOOL

Data contained in each section of the SDS

by Maryvonne GUIDON-LE BRUN

For each section, this tool allows you to identify the data you'll find there.

Warning: the layout of the SDS is taken from REACh Annex II. However, as the exposure scenario is not codified, the references are those of ECHA Guides: Guidance on information requirements and chemical safety assessment / Part D: Exposure scenario development, May 2008 (version 1.1). These guides are not mandatory and are currently being revised. However, a number of suppliers have adopted these formats for the preparation of exposure scenarios. We have therefore retained the references of this plan to create this tool.

MSDS map

Techniques de l'Ingénieur

Warning: the layout of the SDS is taken from REACH Annex II. On the other hand, as the exposure scenario plan is not codified, the references are those of ECHA Guides: Guidance on information requirements and chemical safety assessment / Part D: Exposure scenario development of August 2016 (version 2.0). These guides are not mandatory and are currently being revised. However, a number of suppliers have adopted these formats for developing exposure scenarios. We have therefore kept the references of this plan to produce this tool.

| MSDS map | Supplements |
| --- | --- |
| SECTION 1: Identification of the substance/mixture and of the company/undertaking |  |
| 1.1 Product identifier according to art. 18 of CLP | IUPAC name (or trade name for a mixture) CASE EC N°: EINECS, ELINCS, INDEX NO. (CLP) Registration number if existing or justification if absent |
| 1.2 Relevant identified uses of the substance or mixture and uses advised against |  |
| 1.3. Information concerning the supplier of the safety data sheet | Company name, address, telephone, e-mail, representative in Europe |
| 1.4. Emergency number |  |
| SECTION 2: Hazard identification |  |
| 2.1. Classification of substance or mixture | According to CLP |
| 2.2. Labelling elements | pictogram(s) of danger, warning statement(s), hazard statement(s) and precautionary statement(s). |
| 2.3. Other hazards | Specifies PBT or vPvB, formation of atmospheric contaminants during curing or processing, dusting, risk of dust blows, cross-sensitization, asphyxiation, freezing, very powerful sensitization of smell or taste, environmental effects, e.g. soil organisms or photochemical ozone creation potential. |
| SECTION 3 : Composition/information on ingredients |  |
| 3.1 Substances | chemical identity of any impurity, stabilizing additive or individual component other than the main component, which is itself subject to classification and which contributes to the classification of the substance, Substance suppliers may also list all components, including non-classified ones. This sub-heading can also be used to communicate information on multi-component substances. |
| 3.2 Mixtures | Product identification "mandatory and optional (if desired by the supplier)". Mass concentrations or concentration ranges |
| SECTION 4: First aid |  |
| 4.1. Description of first aid measures |  |
| 4.2. Main symptoms and effects, acute and delayed |  |
| 4.3 Indication of any immediate medical attention and special treatment needed |  |
| SECTION 5 : Fire fighting measures |  |
| 5.1. Extinguishing media |  |
| 5.2. Special hazards arising from the substance or mixture |  |
| 5.3. Advice for fire-fighters |  |
| SECTION 6: Accidental release measures |  |
| 6.1. Personal precautions, protective equipment and emergency procedures |  |
| 6.2. Environmental precautions |  |
| 6.3. Methods and equipment for containment and cleaning up |  |
| 6.4. Reference to other sections |  |
| SECTION 7 : Handling and storage |  |
| 7.1. Precautions for safe handling |  |
| 7.2. Conditions for safe storage, including any incompatibilities |  |
| 7.3. Specific end use(s) |  |
| SECTION 8: Exposure controls/personal protection |  |
| 8.1. Control parameters | Exposure limit values of binding and non-binding national regulations (OEL) Biological limit values of binding and non-binding national regulations For relevant routes of exposure : DNEL and PNEC |
| 8.2. Exposure controls |  |
| 8.2.1. Occupational exposure controls | Collective and individual protective equipment |
| 8.2.2. Environmental exposure controls | Means of preventing exposure |
| SECTION 9: Physical and chemical properties |  |
| 9.1 Information on essential physical and chemical properties | a) Appearance: The physical state (solid - with, where appropriate, suitable safety information available on particle size and specific surface, liquid or gaseous) and color of the substance or mixture; b) smell : c) olfactory threshold ; d) pH: as supplied, or from an aqueous solution; in the latter case, the concentration must be specified; e) melting/freezing point ; f) initial boiling point and boiling range ; g) flash point ; h) evaporation rate ; i) flammability (solid, gas) ; j) upper/lower flammability or explosive limits ; k) vapour pressure ; l) vapour density ; m) relative density ; n) solubility(s) ; o) Partition coefficient: n-octanol/water ; p) auto-ignition temperature ; q) decomposition temperature ; r) viscosity ; s) explosive properties ; t) Oxidizing properties. |
| 9.2. Other information | miscibility, liposolubility (solvent - oil to be specified), conductivity or gas group ; redox potential, free radical formation potential and photocatalytic properties. |
| SECTION 10: Stability and reactivity |  |
| 10.1. Reactivity |  |
| 10.2. Chemical stability |  |
| 10.3 Possibility of dangerous reactions |  |
| 10.4 Conditions to avoid |  |
| 10.5. Incompatible materials |  |
| 10.6 Hazardous decomposition products |  |
| SECTION 11: Toxicological information |  |
| 11.1 Information on toxicological effects | Information and test data used to classify the hazards of the substance or mixture Study summaries/test methods CMR evaluation No data or irrelevant data Justification for non-classification LC50, LD50 Information on likely routes of exposure Techniques de l'Ingénieur Symptoms related to physical, chemical and toxicological characteristics Delayed and immediate effects, and chronic effects of short- and long-term exposure Interactive effects No specific data |
| SECTION 12: Ecological information |  |
| 12.1. Toxicity | LC50 |
| 12.2. Persistence and degradability |  |
| 12.3. Bioaccumulative potential |  |
| 12.4 Mobility in soil |  |
| 12.5. Results of PBT and vPvB assessments |  |
| 12.6. Other adverse effects |  |
| SECTION 13 : Disposal considerations |  |
| 13.1. Waste disposal methods |  |
| SECTION 14: Transport information |  |
| 14.1 UN number |  |
| 14.2. United Nations shipping name |  |
| 14.3. Transport hazard class(es) |  |
| 14.4 Packaging group |  |
| 14.5. Environmental hazards |  |
| 14.6. Special precautions to be taken by the user |  |
| 14.7. Transport in bulk according to Annex II of Marpol 73/78 and the IBC Code |  |
| SECTION 15: Regulatory information |  |
| 15.1 Safety, health and environmental regulations/legislation specific to the substance or mixture | - Indicates information relating to relevant Community provisions on safety, health and the environment (e.g. SEVESO category/designated substances listed in Annex I to Council Directive 96/82/EC ( 1 )). - provided national information on the regulatory status of the substance or mixture (including substances present in the mixture), advice on measures that the recipient should take into account - national laws of the Member States concerned implementing these provisions, as well as any other national measures that may be relevant. - specific provisions concerning the protection of human health or the environment at Community level (e.g. authorizations granted under Title VII or restrictions applied under Title VIII) |
| 15.2. Chemical safety assessment | specifies whether a chemical safety assessment has been carried out by the supplier |
| SECTION 16: Other information | any information not contained in sections 1 to 15, including information relating to the revision of the safety data sheet, and in particular : (a) in the case of a revised SDS, a clear indication of where changes have been made to the previous version of the data sheet, unless this indication is given elsewhere in the data sheet, with an explanation of the changes where appropriate. b) the meaning of abbreviations and acronyms used ; c) main bibliographical references d) in the case of mixtures, an indication of the methods used to evaluate the data referred to in Article 9 of CLP for the purposes of classification; e) a list of H entries, P entries.... The text of the entries reproduced in part in sections 2 to 15 must appear here in full; f) advice on appropriate training for workers to ensure the protection of human health and the environment. If, the supplier of a mixture chooses to identify and communicate the classification required from June 1, 2015 before applying it for classification and labeling purposes on the packaging, he may include this classification in this section. Techniques de l'Ingénieur |
| APPENDIX / EXPOSURE SCENARIOS |  |
| 1 Brief title of the exhibition scenario |  |
| 2 Processes and activities covered by the exposure scenario |  |
| Operating conditions of use |  |
| 3. Duration and frequency of use | Specify for workers, consumers, environment (if applicable) |
| 4.1 Physical form of substance or preparation; surface-to-volume ratio of articles | Gas, liquid, powder, grains, massive solids Surface area per quantity of articles containing the substance (if applicable) |
| 4.2 Concentration of the substance contained in the preparation or article |  |
| 4.3 Quantity used per unit of time or per activity | Specify for workers, consumers, environment (if applicable) |
| 5 Other important operating conditions | For example - temperature, pH, mechanical energy input - capacity of receiving environment (e.g. sewage or river flow rate; room volume x ventilation rate) - wear and tear on items (where applicable); conditions relating to the useful life of items (where applicable) |
| 6 Risk management measures |  |
| 6.1 Risk management measures for human health (workers or consumers) | Type and efficacy of individual options or combinations of options to be quantified with respect to exposure [options should be formulated as informative guidance]; specify for oral, inhalation and dermal routes |
| 6.2 Environmental risk management measures | Type and effectiveness of individual options or combinations of options to be quantified [options should be formulated as informative advice]; specify for wastewater, waste gas, soil protection |
| 7 Waste management measures | at the various stages of the life cycle of substances (including preparations or articles at the end of their useful life); |
| Information on estimated exposure and advice to DU |  |
| 8 Exposure estimates and sources | Estimated exposure resulting from the conditions described above (entries 3 to 7 and properties of the substance; mention the exposure assessment tool used; specify for the different routes of exposure; specify for workers, consumers, environment) |
| 9 Tips for DUs on assessing whether they are working under the conditions prescribed in the ES | Advice on how DU can assess whether it is operating within the conditions set out in the exposure scenario. This can be based on a series of variables (and an algorithm |

Techniques de l'Ingénieur