Monitoring plan

This form serves as a template for drawing up a monitoring plan. It contains examples to illustrate how to fill it in. These elements are to be removed when using it.

Once completed, it forms part of the manufacturing file for the product or service (or family of products or services) associated with the process from which it results.

by Jean DE LA HOUPLIERE, Olec KOVALEVSKY

TOOL

|  |  | Site de |  |  |  | Paris |  |  |  |  |  | MONITORING PLAN |  |  |  |  |  |  |  |  |  |  | Page number |  |  |  | 1 from 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Number: ...auto_453... .......... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Prototype |  | X |  | Pre-series |  |  |  | series | Editor: |  |  | JdLH |  |  | Creation date : |  |  | 20/05/2022 |  | Date modified : |  |  |  |  | 22/05/2022 |  |  |
| Market / Customer |  |  |  | : Auto |  |  |  |  |  | Main customer contact |  |  |  |  | Customer approval if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product number |  |  |  | V-31.888 |  |  |  |  |  | Gilles B. 04 30 80 06 99 |  |  |  |  | Nam |  | function |  |  |  |  | Date |  |  | Visa |  |  |  |  |
| Product index |  |  |  | A |  |  |  |  |  |  |  |  |  |  | Gilles B. |  | AQF |  |  |  |  |  |  |  |  |  |  |  |  |
| Product name |  |  |  | : Valve BVA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Techniques de l'Ingénieur

|  |  |  | Features |  |  |  |  | Control plan |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oper.n° | Operation description | Machine / tool | N° | Product | Process parameter | Character. special | Specification+ tolerance | Measurement/error prevention technology | Sample |  | Document used (confidential) | Reaction plan and corrective actions |
|  |  |  |  |  |  |  |  |  | Size | Frequency |  |  |
| 10 | Piston machining | Meteor |  | Surface finish |  |  | Rz 1.2 | Contour graph | 5 | 1 x / hour | Control board | if Cpk < 1.33, apply MSP rules. |
| 10 | Piston machining | Meteor |  | Aspect |  |  | Next range | visual | 5 | 1 x / hour | Instruction n° yy | Blocking + sorting at 100% of the lot produced since last check OK |
| 200 | Pairing | MASSA transfer line |  | game |  |  | 4 to 7 microns | Integrated probe | 100% | unitary | Manual MASSA line |  |
| 200 | Pairing | MASSA transfer line |  |  | Sensor calibration |  | Valid calibration" message | Stallion | 5 sets / tray | 1 x / shift + each time a new version is released | Instruction n° xx | Stop the line and alert the hierarchy for analysis |
| 200 | Pairing | MASSA transfer line |  | cleanliness |  | ◊ | Max. 500 particles / piece | According to analysis protocol |  | 1st lot of the month | Cleanliness audit | Cleanroom maintenance |
| 200 | Pairing | MASSA transfer line |  |  | Hygrometry |  | 80% max. | Auto. sensor | Not applicable | 1 x / team | Logbook | Alerting management |
| 200 | Pairing | MASSA transfer line |  |  | Temperature |  | 20° +/- 1 | Auto. sensor | Not applicable | 1 x / team | Logbook | Alerting management |
|  |  |  |  |  |  |  |  |  |  | Techniques de l'Ingénieur |  |  |

Format n°: TI 0525.......................... version of 22/05/2022

Techniques de l'Ingénieur

Techniques de l'Ingénieur